Methods

Participants and data collection: This study includes 15 patients with intractable epilepsy undergoing invasive SEEG monitoring at the Barnes-Jewish Hospital epilepsy monitoring unit (Saint Louis, MO) [age and gender ranges]. Patient electrophysiology was recorded using a 256-channel clinical system at a sampling rate of 2 kHz. Data collection and event logging were managed using BCI2000 [cite]. SEEG electrodes [company and description] were composed of 4-18 contacts with 1.5-mm insulation between them. Patients included in this study had at least one electrode in the PCC, MCC, or ACC. Patients with major lesions or prior resections were not included in this study. Patients unable to provide informed consent or who refused to give informed consent were not recruited for this study.

Electrode localization and patient brain segmentation: Patient-specific brain models were generated using pre- and post-operative T1-weighted structural MRI, computer tomography scans, FreeSurfer, and the VERA colocalization software [cite]. The Destrieus Atlas (2009), which includes a cytoarchitectonic and functional parcellation of the cingulate sulci and gyri was used for cortical parcellation [cite https://surfer.nmr.mgh.harvard.edu/fswiki/DestrieuxAtlasChanges]. Electrode localization was performed using VERA, which performs an automated cluster-based detection of electrodes using CT scans, with manual post-detection alignments and adjustments (see VERA documentation [cite]). For the standardization of anatomy and electrode locations, brain models and electrodes were projected to MNI space. Electrodes were labeled based on the anatomical label of parcellation nearest to the electrode coordinate center. Electrodes localized in CC subregions were selected a priori for SPES. Electrode pairs for SPES were then defined as any pair comprising at least one electrode in a CC subregion.

Intracranial SPES: To evoke cortical responses to stimulation of the CC subregions across the cortex, bipolar SPES was delivered to each CC subregion. SPES was delivered at 6 mA at 0.5 Hz, using a 0.2-ms biphasic square pulse. These parameters have previously been validated in both clinical mapping procedures and studies of effective connectivity in humans [cite]. Stimulation was delivered using a Nihon Kohden MS-120B-K-EEG cortical stimulator in conjunction with Nihon Kohden PE-210AK software (Nihon Kohden, Japan). Single pulses were delivered across each preselected electrode pair (dipole) localized within the CC for each patient. SPES was delivered for approximately two minutes for 60 stimulation events (trials). Stimulation events were logged using g.TRIGBOX (g.tec medical engineering, Austria) connected via a 3.5 mm tip-sleeve cable connected to the cortical stimulator.

Data Preprocessing: Processing of electrophysiological data and statistical analysis was conducted in MATLAB environment (Mathworks, Nattick, MA, USA). SEEG data were preprocessed by first removing the stimulation artifact across all trials. The onset of stimulation was determined using the logged stimulation events recorded through the g.TRIGBOX. Stimulation artifacts were removed from the SEEG recordings using two-way interpolation. For each channel and stimulation event, we replaced the signal within a 15-sample window around stimulation onset with an interpolated signal derived from time-reversed pre- and post-stimulation segments, weighted by complementary ramp functions [cite Crowther LJ, Brunner P, Kapeller C, Guger C, Kamada K, Bunch ME, et al. A Quantitative Method for Evaluating Cortical Responses to Electrical Stimulation]. Next, a 5th-order Butterworth high-pass filter with a 0.5 Hz cutoff frequency was applied to remove slow drift. A 60 Hz line noise and its harmonics at 120 Hz and 180 Hz were removed using a 2nd-order infinite impulse response notch filter.

To improve the spatial specificity of recordings, a small Laplacian re-referencing method was used. The signal at each contact was re-referenced by subtracting the average signal recorded from all contacts within a 5-mm radius. The data were then epoched into 1900-ms trials centered about each stimulation event. Finally, the data were normalized by z-scoring to a 900-ms pre-stimulation window taken from the start of the trial to 50-ms before the stimulation event (to avoid any potential contamination from stimulation events).

Data Processing and Statistical Analysis: A cohesive description of each data processing step and associated statistical analysis is provided in Supplemental Methods. In the present work, we defined two measures of effective connectivity: response coherence and response magnitude. Response coherence was used as a metric to quantify SPES-evoked changes in LFP morphology at each contact (Figure 2a-2b). This was done by comparing the distributions of inter-trial pairwise Spearman Correlations between pre- and post-stimulation time windows. The pre-stimulation (henceforth referred to as baseline) window was defined by an 850-ms window from the start of each trial, and the post-stimulation (henceforth referred to as CCEP) window was defined as a 700-ms window following stimulation onset. The Cohen’s d between these distributions defined the effect size of effective connectivity as measured by response coherence. We defined contacts as significantly effectively connected to the stimulated region